*****   1541-Software-Emulator by Dennis Kuschel Productions 1995   *****

With this pack you've got the first well-running version of the

floppydrive Commodore 1541-emulation. Whith a self-made cable (see the

cable.doc file) you can connect the PC to the master-device (e.g. C16/

C64/C128/Plus4). The emulator is a real software-emulator, that means

the emulated drive has no real or emulated 6502-CPU. The consequences are

that no software-speeder or other m-code will run (ohh! The Comodore-

serial-handling is hard enought... please no speeder!!!!).

To avoid communication-errors please don't run this programm in a multi-

tasking-area and dispose all disccaching-programs and so on from memory!

But in fact communication-errors will sometimes occour (sorry, but the

hardware-timing... ). The emulator runs well at a 80386 with 40MHz and

MS-DOS 6.0 installed. If the CPU-clock is too lo (<33MHz), communication-

errors will occour automaticaly. But in some cases the CPU might be too

fast, and the communication doesn't work correctly, so you have do slow

down your PC and/or choose a slower speed in the main-menu by pressing 's'.

You can use this emulator as stand-alone device or with other floppy-

devices, but note, if it is not used as stand-alone device, the other

devices have to be turned on! (cased by the open-collector-cablehandling =>

pull-up-resistors will be pull-down-resistors if a device is turned off)

- The disc-images are fully compatible with the C64s-emulator form

Miha Peternel, so you can exchange them.

remember, this is the first well-running program-version!

This program is card-ware, that means you can copy it freely without changing

in it and please write my a postcard. If you have troubles with these program

or you want to have an adapter-cable, note it.  My address:

Dennis Kuschel, Brnyweg 1, 28277 Bremen, Germany.

This version was tested and runs well on a 486/40 under raw MSDOS5.0,

MSDOS 6.2 (with himem, smartdrive and all...) and (--->) OS/2 in DOS-BOX!!!

ans a PLUS4 with 256K on the other side of the cable...........(:-o)

WBr